Omega Race Position Encoder

By Bryan Edewaard

When I bought my Omega Race machine, I was rather disappointed by the feel of its rotation knob - not that the knob felt particularly bad, but that the ship didn’t respond in a linear way.   Near the top of  the ship’s rotation, the knob was very touchy; a small turn on the wheel would cause a very  big turn in the ship.  Nearly  three-quarters of  the ship’s rotation occurred in one-half of the knob’s rotation.

The reason?  Midway found the 64-position encoder wheels they were using to be in short supply and substituted a 360 degree pot and an A/D converter board.  Although this solution worked, it was very imprecise even with careful tweaking of the A/D circuit, and the location where the pot wraps (goes from max to min resistance) would cause all kinds of unpredictable behavior.  For a sick gamer like me, it was time to go to work.

This is a simple circuit that someone with access to an EPROM burner, a soldering iron, and some common tools can build in an afternoon.  The concept is simple and very similar to an optomechanical mouse.  Two optosensors are placed where they can read the light/dark boxes around the outside of the wheel.  It does not matter where the optosensors are placed as long as they are 90 degrees out of phase with each other.  This may take some manual tweaking of position, but should be easy to accomplish.  As the wheel turns, the sensors will return the following sequence:

Forward:  0,0  1,0  1,1  0,1  0,0...    Backward:   0,0  0,1  1,1  1,0  0,0...

This sequence is fed into the address lines of  an EPROM, and a count value is returned on the data pins which is also fed back into address pins so the EPROM can return count values based on the previous count.  By using an EPROM, a great deal of extra circuitry was eliminated and the EPROM can compensate for the out-of-order count sequence used by the original encoder.

Parts list:

2 optosensors

These are available from many surplus electronics houses at an inexpensive price.  Get the type

that features both an LED and detector in one slotted assembly with screw down mount.

2732 EPROM

Although only a 2708 is required to hold the data, they are becoming very difficult to find and

2732’s are cheap.  If you use a 2708 or 2716, just truncate the ROM file after the 1024 or 2048

bytes since it contains redundant data.  Speed is not a factor here and 400ns EPROM’s will work

fine.

74LS74 Dual D Flip-Flop

This is used to ‘debounce’ the circuit to make sure the EPROM does not see changes on its

address lines during while it looks up data.

74LS374 8-bit Latch

This is used to latch the current state that will be sent to Omega Race and back into the address

lines of the EPROM.

555 Timer IC

This is the sampling frequency generator.

2N2923 / MPS2222A (RS cat. no. 276-2009) NPN transistor

Used as a quick & dirty inverter to avoid another IC.

Capacitors:  10uF (10v to 35v) electrolytic, .015uF ceramic

Resistors:  100K, (2)4.7K, (3)10K, (2)220, all 1/4 watt, and any unneeded potentiometer

Misc:  Plastic butter tub lid (see text), Perfboard

Remove the original pot from Omega Race by loosening the set screws on the shaft and removing the c-clip that holds the back of the pot bracket.  Remove the pot from the bracket and unplug the pot’s harness from the A/D converter board and the control panel harness and remove.

Print the wheel from page one onto a laser printer transparency and carefully cut it out.  Using a black permanent fine-tip marker, make sure the black squares around the outside are fully filled in by holding the wheel up to a bright light and darkening any light areas with the marker on the printed side of the transparency.

Cut out a piece of stiffer plastic (I used a butter tub lid) about 3/8” radius smaller than the wheel and glue it to the center of  the wheel to provide stiffness.  I used a spray adhesive, but a super-glue or epoxy would work.  Make sure no glue gets on the outer exposed part of the wheel or it could interfere with the optosensors.

Next, you will need to remove the back of the potentiometer.  The wheel will need to be connected to the pot shaft in a reliable and aligned manner.  This may involve gluing the wheel to internal parts, or putting the shaft through the wheel.  On my pot, the wiper assembly was held in by a metal tab extending from the back of the shaft which I was able to put through the wheel with a few nylon washers behind it, but this part I pretty much have to leave up to your ingenuity.  When you’re done, you should have a mountable wheel assembly with shaft.  Make sure the shaft is between .5” and 1” in exposed length or it may not mate to the knob assembly properly.

Mount the wheel assembly centered in an approximately 3”x3” square of perfboard with the metal bracket the old Omega Race pot is attached to.  Next mount the 2 optosensors on the perfboard (preferably in 2 corners of the board) with the wheel edge in the slot of the optosensor.  Mount them at a distance where the middle of the outer ring of the wheel is approximately half-way into the slot.  Make sure the screw holes in the perfboard are big enough to allow one of the optosensors to slide sideways enough to align them later.

Next you will need to construct the following circuit.  I recommend using a socket in place of the EPROM since it is a fairly expensive and fragile component (also because you may not have programmed it yet).

The circuit has basically three parts: clock circuit, input sampling and sequence generator.  The two inputs from the optosensors are fed into a 74LS74 Flip Flop which relatches their states at about 1KHz.  This is the sampling circuit and it guarantees that the data being fed to the address lines of the EPROM will remain constant while the output is being read.  The sequence generator uses a pre-programmed EPROM and returns the current position by comparing the previous position, the previous optosensor state, and the current optosensor state.

The clock circuit is a simple 555 timer circuit and an inverted clock is provided by a simple transistor circuit.

When you connect this circuit to the machines harnesses,  I recommend that you use the harnesses that are used to connect to the A/D board to the control panel input harnesses.  The input harness is the 10 position flat connector, the power harness is the 6 position (3x2) harness.

Once the board is assembled and connected, try turning the wheel slowly to check its operation.  If you get no response from the game, re-check your connections and circuit.  If  the ship rotates the wrong way, reverse your sensor 1 and sensor 2 input connections.  If you discover that the ship turns, but doesn’t operate reliably if you turn the wheel quickly, then it’s time to align the optosensors.

Connect the signals from pins 5 and 9 of the 74LS74 to 2 LEDs in series with a 220 ohm resistor to ground to watch the optosensor states change.  As you turn the wheel, you’ll notice the lights going on and off.  Move one of  the optosensors side to side until the spacing between changes in state appears to be even.  This doesn’t have to be exact, but if it’s too far out of whack, the input sampling circuit will miss transitions that happen too close together.

Once the new sensor is assembled and adjusted, mount the sensor wheel board by putting the mounting bracket on the post with the c-ring and attach the sensor’s shaft to the input knob.  Put the control panel back in and enjoy!

If you have any comments or find any errors in this document, you can reach me at bryedewaard@delphi.com.